xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

Network Set-up

xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

Squeezecenter Laptop 1 (192.168.1.40) & Receiver are connected to same switch (a NetGear Fast Ethernet switch). Homeplug is still on the network – but no Squeeze equipment sits behind anymore.
Controller is connecting to network via Receiver  (as I do not want to depend on my home WLL – which I only switch on when needed). The Duet is essentially there to feed my living room HiFi from a NAS.

Wireshark is running on a different laptop 2 (192.168.1.38)

A 3rd Laptop (192.168.1.33) is ready to send a Magic Packet to Squeezecenter Laptop 192.168.1.40

xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

Test sequence

xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

Playing Music OK

Press PAUSE on Controller. Music STOPS.
Manually SUSPEND Laptop.

Starting Wireshark

Moving Controller as screen is black.
Press PAUSE again to continue PLAY 
Display  >>> “Connecting To Laptop 1…”
After few secs… it displays : “Try Again or Choose Musice Source” “ There is a pb connecting to Laptop 1”

Pressing “Choose Music Source” then searches again…

“Laptop 1” or “SqueezeNetwork” alternative shown again 

Trying again on Laptop 1…

Searching / no connection found / Laptop not waking.

Wireshark does not see any WOL packet so far.

Finally – WOL sent from Laptop 3.

Wireshark detects it & Laptop wakes.

Music starts playing rightaway to my surprise. Controller still seems a bit lost. 

Stopping Wireshark Record on laptop 3. 

On Controller … choosing “Connecting to Squeezebox” – display on Controller is finally fine and in line with music that continued to play since wake.
xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

Software versions involved
xxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxxx

On Controller  about:
Version 7.1r2722 Mon Jul 21

MAC address 00:04:20:1A:1A:86

On Squeezebox information:

Model : receiver

Firmware 36

IP : 192.168.135:24343

MAC : 00:04:20:16:1D:95

On Squeezecenter: SqueezeCenter Information

SqueezeCenter Version: 7.1 - 22170 @ Mon Jul 28 13:36:57 PDT 2008 - Windows XP - EN - cp1252
Server IP address: 192.168.1.40
Perl Version: 5.8.8 MSWin32-x86-multi-thread
MySQL Version: 5.0.22-community-nt

Platform Architecture: 586
Hostname: Santana
Server Port Number: 9000
Total Players Recognized: 1
 
Player Information

Name: Squeezebox
Model: receiver
Firmware: 36
The IP address for this player is: 192.168.1.35:24345
The Ethernet MAC address for this player is: 00:04:20:16:1d:95
Wireless Signal Strength: 94
WOL manual sent :

[image: image1.png]He

Mac Address
Internet Address
Subnet Mask.

Send Options

¥ Wake On Lan - Magic Packet

000656573440
192168.1.40
266.266.266.255

Local Subret g

Remate Port Number g

8 /ake Me Up

s [Magic Packet Sent To 255 265 255 255





